- Discuss block diagram

  - State how input signal is about 1.4V amplitude

  - Produced from 3.5mm audio jack

- Preamp stage

  - Using Baxandall 3-band config

  - Will allow for attenuation of low, mid, high frequencies

  - Volume control

  - Gain of about 1

  - Using NE5532, industry standard audio dual op amp with low noise

  - Not rail to rail, but with 12V supply it will have enough headroom

  - Design will be refined through breadboard

  - Will eventually require bespoke pcb

- Amplifier Stage

  - Will use off the shelf amplifier chip

  - Will use powerful one and underdrive it for more headroom to reduce distortion

  - Class AB is a design using push/pull transistor config, commonly used for audio designs due to low distortion

  - Must have enough headroom in output to prevent THD of 10% at 5W

- Power constraints: Due to DC only design, we don’t have the ability to provide dual voltage rails.

  - With a 12V supply, will bias around 6V and use DC decoupling capacitors for output